# EsaBend IoT ModBusTCP P2

NOTE 1: Some of the features described may not be present on older versions of EsaBend. NOTE 2: ver. only 58.1 of EsaBend contains all the features described.

## Description

EsaBend is enabled to make a series of data visible and receive a series of data and commands from an external supervisor which can reside on the same PC as EsaBend or on a different PC.

To make a series of data visible and receive data and execute commands EsaBend uses a ModBusTCP server; the supervisor must therefore be a ModBusTCP client that reads data and transmits data and commands via the appropriate ModBus registers.

## Enable feature

The EsaBend ModBusTCP server service is paid feature and must be enabled via KvOption: if the checkbox in "MODBUS TCP" is enabled, the service has been enabled.

Copy file from: C:\Esabend\Kvara\Exe\NewXMLModBus\ServerModbus.xml To

C:\Esabend\Kvara\Exe\ServerModbus.xml Open ServerModbus.xml and configure correct IP in:

<IPAddr>127.0.0.1</IPAddr>

In Basic Parameter\Tandem Evolved set the parameter to 2 TE3

To check if the ServerModBusTCP is active, check that the 2 messages have been printed in PLC Menú:

The commands arriving from the supervisor are "heard" by EsaBend from the "Classic" numerical setting pages of the work program.

- SupervisorClientModBusTCP<->EsaBend architecture

The components that make up the system are:

  - EsaBend + ServerModbus.dll

  - Supervisor + ClientModBusTCP made by third parties

The data and commands written\read by the ModBusTCP client supervisor are read\written by the ModBusTCP server ServerModbus.dll which then writes\reads in shared memory; the maximum limit of registers read in block is 128; EsaBend reads\writes in shared memory and updates the data and executes commands received from the supervisor; the figure shows the “Load program” request sent by the Supervisor to EsaBend:

## Binding from\to ModBus register ena EsaBend cnc shared memory register

The binding from\to ModBus register ena EsaBend cnc shared memory register mapped in C:\Esabend\Kvara\Exe\NewXMLModBus\ServerModbus.xml file.

For example:

- the bending angle of actual step in shared memory is in “progS.angyaS” and in “1253-1256” ModBus register; the binding in ServerModbus.xml file is in input ModBus are:

<Field Priority=0 Size=4>

<MbAddr>1253</MbAddr>

<ShAddr>progS.angyaS</ShAddr>

</Field>

- the command operating code of EsaBend IoT protocol in shared memory is in “EBCS.wCommandFC” and in “1” ModBus register; the binding in ServerModbus.xml file is in output ModBus are

<Field Priority=0 Size=1>

<MbAddr>1</MbAddr>

<ShAddr>EBCS.wCommandFC</ShAddr>

</Field>

## Mapping

Address Modbus = number of ModBus register to read EsaBend data, write data to EsaBend and give commands to Esabend

Description = description of the data read or written or of the command executed

N.Register 16 bit = number of ModBus register

Client Output:

| Address Modbus | N.Register 16 bit | Description |
| --- | --- | --- |

| 0 | 1 | Strobe selection of operating modes, BIT0 Editor BIT1 Manual BIT2 Semi-automatic BIT3 Automatic |
| --- | --- | --- |
| 1 | 1 | BIT0 strobe command |
| 2 | 2 | Command opcode |
| 4 | 1 | Given if type byte |
| 5 | 1 | Given if type word |
| 6 | 1 |  |
| 7 | 2 | Given if type dword |
| 9 | 4 | Dato se tipo double |
| 53 | 40 | Given if type string |

Output EsaBend Server:

| Address Modbus | N.Register 16 bit | Description |
| --- | --- | --- |
| 1024 | 2 | Error code |
| 1026 | 100 | Error description header |
| 1126 | 100 | Error description |
| 1226 | 1 | Acknowledgement operative modes: BIT0 Editor BIT1 Manual BIT2 SemiAutomatic BIT3 Automatic |
| 1227 | 1 | Active operating mode: BIT0 Editor mode active BIT1 Manual mode active BIT2 Semiautomatic mode active BIT3 Automatic mode active |
| 1228 | 1 | Windows popup error code |
| 1229 | 2 |  |
| 1231 | 1 | BIT0 acknowledgement command |
| 1232 | 1 | Type of bend |
| …. | …. | …. |
| …. | …. | … |
| 1965 | 82 | Program name |

The data referring to the fields of the current bend are mapped from ModBus register 1232 to register 1965

| Address Modbus | N.Register 16 bit | Description |
| --- | --- | --- |
| …… | …… |  |
| 1232 | 1 | Type of bend |
| 1233 | 1 | Rotation |
| 1234 | 1 | Number of times the bend must be repeated before passing on to the next one |
| …. | …. | … |
| 1253 | 4 | Bending angle required |
| ……. | …… |  |
| 1965 | 82 | Program name |

Look in ServerModbus.xml to all bend data mapped in ModBus register.

## Command operating codes

COMMAND_WORK_DIR	3001

COMMAND_RESET_WIN_POPUP_ERR	3002

COMMAND_SAVE_PROG	3003

COMMAND_CONNECT_DISAB_MOUSE_MACHINE2 3004

COMMAND_CONNECT_MACHINE2	3005

COMMAND_DISCONNECT_MACHINE2	3006

COMMAND_STOP	3007

COMMAND_START	3008

COMMAND_LOAD_WORK_LIST	3009

COMMAND_DELETE_PROG	3010

COMMAND_COPY_PROG	3011

SHUT_DOWN	3012

COMMAND_NEXT	3013

COMMAND_PREV	3014

COMMAND_NEW_PROG_NUM	3015

COMMAND_SUPPORT	3016

COMMAND_DUTCH	3017

COMMAND_CHANGE_SECTION	3018

COMMAND_INCREMENTAL	3019

COMMAND_COINING	3020

COMMAND_UPDATE_PROG	3021

COMMAND_DELETE_SECTION	3022

COMMAND_CHANGE_PRESS_MODE	3023

COMMAND_ACTBOMB	3024

COMMAND_SAVE_AS_PROG	3025

COMMAND_BUMPING	3026

COMMAND_DEL_BEND	3027

COMMAND_CONIC	3028

COMMAND_NUM_BEND_SEC	3029

COMMAND_LOAD_PROG	3030

COMMAND_LEARN_Y	3031

COMMAND_LEARN_Z	3032

COMMAND_CHANGE_PROGRAM	3033

COMMAND_CHANGE_PROGRAM_BG	3034

COMMAND_CORR_BY_SEC	3035

COMMAND_CORR_BY_PIECE	3036

COMMAND_CHANGE_BEND_METHOD	3037

COMMAND_CHANGE_BEND_METHOD_EQUAL_ANGLES 3038

COMMAND_CHANGE_BEND_METHOD_ALL_PIECE	3039

COMMAND_LEARN_BEND_SENS_ANG	3040

COMMAND_LEARN_BEND_EQUAL_ANGLES_SENS_ANG 3041

COMMAND_CHANGE_OFFSET_POS_PEZZO	3042

COMMAND_CHANGE_OFFSET_POS_PEZZO_BY_SEC	3043

COMMAND_CHANGE_OFFSET_POS_PEZZO_BY_PIECE 3044

COMMAND_RUN_LASERCHECK	3045

COMMAND_INIT_COMM_LASERCHECK	3046

COMMAND_SEND_BEND_PARAM_LASERCHECK	3047

COMMAND_SEND_SET_TOOL_LASERCHECK	3048

COMMAND_SCAN_ANGLES	3049

COMMAND_SEND_SET_LASER_ON_OFF_LASERCHECK 3050

COMMAND_SEND_CONT_MEAS_ON_OFF_LASERCHECK 3051

COMMAND_SET_FRAME_RATE_LASERCHECK	3052

COMMAND_ECO_REQUEST_LASERCHECK	3053

COMMAND_SAUTO_AUTO_GRAFICO	3054

COMMAND_NUM_BEND_SEC_GRAFICO	3055

## Command operating codes that change program and bend values

HD_PIECEP	0	// programmed pieces HD_PIECEM	 1	 // pieces made

HD_MEASURES	11

HD_DEVELOP	12

HD_NAME_PRG	13

HD_ACT_BEND	14

HD_ACT_SECT	15

HD_NOME	16	// program name

HD_NUM_BEND_SEC		17// currently selected bend number HD_NUMDATI	18		// bend data number in double

1000 must be added to the following codes:

| PG_TYPE | 1000 |  | // | type of step |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PG_ROT | 1001 |  | // | direction of fold |  |  |  |  |  |
| PG_RIP | 1002 |  | // | number of caland step repetitions. |  |  |  |  |  |
| PG_FUN | 1003 |  | // | dword 20 machine functions |  |  |  |  |  |
| PG_NUMDATIB | 1004 |  | // | Number of bend data in bytes |  |  |  |  |  |
| PG_ACCOMP | 1005 |  | // | accompanying speed |  |  |  |  |  |
| PG_THICK | 1006 |  | // | sheet thickness |  |  |  |  |  |
| PG_TBEND | 1007 |  | // | time spent in the fold |  |  |  |  |  |
| PG_CORPIN | 1008 |  | // | correction altitude 180 |  |  |  |  |  |
| PG_ANGY1 | 1009 |  | // | angle y1 |  |  |  |  |  |
| PG_ANGY2 | 1010 |  | // | angle y2 |  |  |  |  |  |
| PG_X1 | 1011 |  |  | // |  | double | axis | x1 |  |
| PG_X2 | 1012 |  |  | // |  | double | axis | x2 |  |
| PG_X3 | 1013 |  |  | // |  | double | axis | x3 |  |
| PG_X4 | 1014 |  |  | // |  | double | axis | x4 |  |
| PG_Y1 | 1015 |  |  | // |  | double | axis | y1 |  |
| PG_Y2 | 1016 |  |  | // |  | double | axis | y2 |  |
| PG_R1 | 1017 |  |  | // |  | double | axis | r1 |  |
| PG_R2 | 1018 |  |  | // |  | double | axis | r2 |  |
| PG_R3 | 1019 |  |  | // |  | double | axis | r3 |  |
| PG_R4 | 1020 |  |  | // |  | double | axis | r4 |  |
| PG_Z1 | 1021 |  |  | // |  | double | axis | z1 |  |
| PG_Z2 | 1022 |  |  | // |  | double | axis | z2 |  |
| PG_Z3 | 1023 |  |  | // |  | double | axis | z3 |  |
| PG_Z4 | 1024 |  |  | // |  | double | axis | z4 |  |
| PG_A1 | 1025 |  |  | // |  | double | axis | auxiliary | 1 |
| PG_A2 | 1026 |  |  | // |  | double | axis | auxiliary | 2 |
| PG_A3 | 1027 |  |  | // |  | double | axis | auxiliary | 3 |
| PG_A4 | 1028 |  |  | // |  | double | axis | auxiliary | 4 |
| PG_A5 | 1029 |  |  | // |  | double | axis | auxiliary | 5 |
| PG_A6 | 1030 |  |  | // |  | double | axis | auxiliary | 6 |
| PG_A7 | 1031 |  |  | // |  | double | axis | auxiliary | 7 |
| PG_A8 | 1032 |  |  | // |  | double | axis | auxiliary | 8 |
| PG_CORAY1 | 1033 | // | correction |  | angle y1 |  |  |  |  |
| PG_CORAY2 | 1034 | // | correction |  | angle y2 |  |  |  |  |
| PG_CORX1 | 1035 | // | correction |  | x1 |  |  |  |  |
| PG_CORX2 | 1036 | // | correction |  | x2 |  |  |  |  |
| PG_CORX3 | 1037 | // | correction |  | x3 |  |  |  |  |
| PG_CORX4 | 1038 | // | correction |  | x4 |  |  |  |  |
| PG_CORY1 | 1039 | // | correction |  | y1 |  |  |  |  |
| PG_CORY2 | 1040 | // | correction |  | y2 |  |  |  |  |
| PG_RINX1 | 1041 | // | retract x1 |  |  |  |  |  |  |

| PG_RINX2 | 1042 |  | // | retract | x2 |
| --- | --- | --- | --- | --- | --- |
| PG_RINX3 | 1043 |  | // | retract | x3 |
| PG_RINX4 | 1044 |  | // | retract | x4 |
| PG_VELW | 1045 |  | // | working | speed |
| PG_TCP |  | 1046 | // | step change time |  |
| PG_PMS |  | 1047 | // | top dead center |  |
| PG_NSTAZ |  | 1048 | // | station number |  |
| PG_CSENS |  | 1049 | // | flag sensor |  |
| PG_SENS1 |  | 1050 | // |  |  |
| PG_SENS2 |  | 1051 | // |  |  |
| PG_SENS3 |  | 1052 | // |  |  |
| PG_SENS4 |  | 1053 | // |  |  |
| PG_SENS5 |  | 1054 | // |  |  |
| PG_SENS6 |  | 1055 | // |  |  |
| PG_METODO |  | 1056 | // |  |  |
| PG_SA |  | 1057 |  |  |  |
| PG_SB |  | 1058 | // |  |  |
| PG_SC |  | 1059 | // |  |  |
| PG_SD |  | 1060 | // |  |  |
| PG_SE |  | 1061 | // |  |  |
| PG_SF |  | 1062 |  |  |  |
| PG_ALTALETTA |  | 1063 |  |  |  |
| PG_NEW5 |  | 1064 |  |  |  |
| PG_NEW6 |  | 1065 |  |  |  |
| PG_NEW7 |  | 1066 |  |  |  |
| PG_NEW8 |  | 1067 |  |  |  |
| PG_NEW9 |  | 1068 |  |  |  |
| PG_GEN_B1 |  | 1069 |  |  |  |
| PG_GEN_B2 |  | 1070 |  |  |  |
| PG_GEN_B3 |  | 1071 |  |  |  |
| PG_GEN_B4 |  | 1072 |  |  |  |
| PG_GEN_B5 |  | 1073 |  |  |  |
| PG_GEN_B6 |  | 1074 |  |  |  |
| PG_GEN_B7 |  | 1075 |  |  |  |
| PG_GEN_B8 |  | 1076 |  |  |  |
| PG_GEN_B9 |  | 1077 |  |  |  |
| PG_GEN_D1 |  | 1078 |  |  |  |
| PG_GEN_D2 |  | 1079 |  |  |  |
| PG_GEN_D3 |  | 1080 |  |  |  |
| PG_GEN_D4 |  | 1081 |  |  |  |
| PG_GEN_D5 |  | 1082 |  |  |  |
| PG_GEN_D6 |  | 1083 |  |  |  |
| PG_GEN_D7 |  | 1084 |  |  |  |
| PG_GEN_D8 |  | 1085 |  |  |  |
| PG_GEN_D9 |  | 1086 |  |  |  |
| PG_SPEED_DEC_LC |  | 1087 |  |  |  |
| PG_DEC_DISTANCE_LC |  | 1088 |  |  |  |
| PG_FIN_BEND_STATUS_LC | 1089 |  |  |  |  |
| PG_Y1PERDEG_LC | 1090 |  |  |  |  |
| PG_Y2PERDEG_LC | 1091 |  |  |  |  |
| PG_SB_Y1_LC | 1092 |  |  |  |  |
| PG_SB_Y2_LC | 1093 |  |  |  |  |
| PG_ANGY1_FIN_LC | 1094 |  |  |  |  |
| PG_ANGY2_FIN_LC | 1095 |  |  |  |  |
| PG_FORCE_MIS_LC | 1096 |  |  |  |  |
| PG_POSY1_FIN_LC | 1097 |  |  |  |  |
| PG_POSY2_FIN_LC | 1098 |  |  |  |  |
| PG_DB_NUM_STOP_LC | 1099 |  |  |  |  |
| PG_DB_QTA_PCL_LC | 1100 |  |  |  |  |
| PG_SB_IMG100 | 1101 |  |  |  |  |

| PG_WARP_IMG100 | 1102 |  |  |
| --- | --- | --- | --- |
| PG_CONFIDENCE_IMG100 | 1103 |  |  |
| PG_NUM_TENTA_IMG100 | 1104 |  |  |
| PG_METODO_IMG100 | 1105 |  |  |
| PG_TOLL_IMG100 | 1106 |  |  |
| PG_PREBEND_IMG100 | 1107 |  |  |
| PG_ANG_FIN_IMG100 | 1108 |  |  |
| PG_ANG_PRE_B_IMG100 | 1109 |  |  |
| PG_VEL_SAL_IMG100 | 1110 |  |  |
| PG_DIE | 1111 |  |  |
| PG_PUNCH | 1112 |  |  |
| PG_SEL_PUNTI_MISURA_LC | 1113 |  |  |
| PG_QTA_PNT_MIS_Y1_LC | 1114 |  |  |
| PG_QTA_PNT_MIS_Y2_LC | 1115 |  |  |
| PG_QTA_PNT_MIS_YB_LC | 1116 |  |  |
| PG_QTA_PARCH_LASER_LC | 1117 |  |  |
| PG_SCAN_ANGLE_Y1_LC | 1118 |  |  |
| PG_SCAN_ANGLE_Y2_LC | 1119 |  |  |
| PG_NUM_NO_MATCH_Y1_LC | 1120 |  |  |
| PG_NUM_NO_MATCH_Y2_LC | 1121 |  |  |
| PG_ANGLE_Y1_FRONT_LC | 1122 |  |  |
| PG_ANGLE_Y1_BACK_LC | 1123 |  |  |
| PG_ANGLE_Y2_FRONT_LC | 1124 |  |  |
| PG_ANGLE_Y2_BACK_LC | 1125 |  |  |
| PG_PREBEND_PMI | 1126 |  |  |
| PG_DIE_HOLDER 1127 |  |  |  |
| PG_INTERMEDIATE 1128 |  |  |  |
| PG_PMI_AFTER_PREBEND 1129 |  |  |  |
| PG_LAB_MIN |  |  | 1130 |
| PG_AREA_SIC_X |  |  | 1131 |
| PG_AREA_SIC_R |  |  | 1132 |
| PG_SENS_PRES_BOMB_ATT 1133 |  |  |  |
| PG_SENS_PRES_FIND_MAX 1134 |  |  |  |
| PG_SENS_PRES_FIND_SPESS |  | 1135 |  |
| PG_WIDTH 1136 |  |  |  |
| PG_V_DIE 1137 |  |  |  |
| PG_WIDTH_V_DIE 1138 |  |  |  |
| PG_FORCE 1139 |  |  |  |
| PG_CROWNING 1140 |  |  |  |
| PG_MUTEPOINT 1141 |  |  |  |
| PG_INNER_RADIUS 1142 |  |  |  |
| PG_NAME_MATERIAL 1143 |  |  |  |
| PG_NUM_MATERIAL 1144 |  |  |  |
| PG_RESISTANCE 1145 |  |  |  |
| PG_YB 1146 |  |  |  |
| PG_CORYB 1147 |  |  |  |
| PG_QTA_PARCH_LASER_DX_LC 1148 |  |  |  |
| PG_SENS_ATTIVI 1149 |  |  |  |
| PG_PMI_AFTER_PREBEND_Y2 1150 |  |  |  |
| PG_ROT1 |  |  | 1151 |
| PG_ROT2 |  |  | 1152 |
| PG_ROT_VEL 1153 |  |  |  |
| PG_COR_ROT 1154 |  |  |  |
| PG_ANGY3 |  | 1155 | // angle y3 |
| PG_ANGY4 |  | 1156 | // angle y4 |
| PG_Y3 |  | 1157 | // double axis y3 |
| PG_Y4 |  | 1158 | // double axis y4 |
| PG_CORY3 |  | 1159 | // correction y3 |
| PG_CORY4 |  | 1160 | // correction y4 |
| PG_ROT_PUNCH 1161 |  |  |  |

PG_ROT_DIE	1162

PG_OFFSET_POS_PEZZO	1163

PG_CORR_ANG_Y1_LC	1164

PG_CORR_ANG_Y2_LC	1165

PG_CORR_ANG_YB_LC	1166

PG_CORR_QTA_PNT_MIS_Y1_LC	1167

PG_CORR_QTA_PNT_MIS_Y2_LC	1168

PG_CORR_QTA_PNT_MIS_YB_LC	1169

## Protocol

### Client Command protocol:

- Write data to one or more ModBus registers address 4 or 5 or 6 or 7 or 9 or 53 (depending on the command)

- Write command opcode in ModBus register address 2

- Write strobe command (bit0 to 1) in ModBus register address 1 (set strobe)

- Wait acknowledgment command (bit0 to 1) in ModBus register address 1231 (wait acknowledgment go 1)

- Write strobe command (bit0 to 0) in ModBus register address 1 (Reset strobe)

- Wait acknowledgment command (bit0 to 0) in ModBus register address 1231 (wait acknowledgment go 0)

The ModBus registers address 4 or 5 or 6 or 7 or 9 or 53 to write to depends on the data type For program and bend data, refer to the "Current bend data" chapter.

### Client operating mode selection protocol

The EsaBend operating modes that can be selected from the protocol are: Settings, Manual, Semi-Automatic, Automatic.

- Write to 1 of the appropriate bit depending on the operating mode to be selected in ModBus register address 0 (set strobe)

- Wait acknowledgment operatign mode for the appropriate bit (bitXX to 1) in ModBus register address 1226 (wait acknowledgment go 1)

- Write to 1 of the appropriate bit depending on the operating mode to be selected in ModBus register address 1 (Reset strobe)

- Wait acknowledgment operating mode for the appropriate bit (bitXX to 0) in ModBus register address 1226 (wait acknowledgment go 0)

### Example of Automatic Operating Mode selection

- Write bit3 to 1 in ModBus register address 0 (set strobe)

- Wait for bit3 to 1 in ModBus register address 1226 (wait acknowledgment go 1)

- Write bit3 to 0 in ModBus register address 0 (Reset strobe)

- Wait for bit3 to 0 in ModBus register address 1226 (wait acknowledgment go 0)

### Example of Load Program command

NOTE: Before sending the load program command, select the Editor operating mode.

- Write program name to ModBus registers address 53-63

- Write command opcode 3030 in ModBus register address 2

- Write strobe command (bit0 to 1) in ModBus register address 1

- Wait acknowledgment command bit0 to 1 in ModBus register address 1231

- Write strobe command bit0 to 0 in ModBus register address 1

- Wait acknowledgment command bit0 to 0 in ModBus register address 1231

## Example changes Pieces to Make

NOTE: Before sending data, select the Editor operating mode.

- Write pices to make to ModBus registers address 7

- Write command opcode 0 in ModBus register address 2

- Write strobe command bit0 to 1 in ModBus register address 1

- Wait acknowledgment command bit0 to 1 in ModBus register address 1231

- Write strobe command bit0 to 0 in ModBus register address 1

- Wait acknowledgment command bit0 to 0 in ModBus register address 1231

## Example change Angle

NOTE: Before sending data, select the Editor operating mode.

- Write angle to ModBus registers address 9

- Write command opcode 1009 in ModBus register address 2

- Write strobe command bit0 to 1 in ModBus register address 1

- Wait acknowledgment command bit0 to 1 in ModBus register address 1231

- Write strobe command bit0 to 0 in ModBus register address 1

- Wait acknowledgment command bit0 to 0 in ModBus register address 1231

## Debug

The shared memory registers involved for the protocol is defined in defcn_EBCS and to debug the protocol signals just use TksShare.exe and connect the shared memory directors listed below:

//EsaBendClientServer typedef struct {

//From Client to Server

word  wMode;	// Strobe selection of operating modes

word wCommandFC;	// strobe command dword dwCodeFC;	// Command opcode byte byDataFC;	// Given if type byte

word  wDataFC;	// Given if type word

word  wDataFC_b;

dword dwDataFC;  // Given if type dword

double dDataFC;	// Given if type double

string strDataFC[40];	// Given if type string

.

.

.

//From Server to Client

dword  dwErrorCode;	// Error code

string strErUITextA[200];	//Error description header

string strErUITextB[200];	// Error description

word	wAckMode;	// Acknowledgement operative modes

.

word	wAckCommandFS;	acknowledgement command

} S_EBCS;

.

S_EBCS EBCS;

the mapping ModBus register and shared memory register is:

Client Output:

| Address Modbus | N.Register 16 bit | Registers in shared memory | Description |
| --- | --- | --- | --- |
| 0 | 1 | EBCS.wMode | Strobe selection of operating modes, BIT0 Editor BIT1 Manual BIT2 Semi-automatic BIT3 Automatic |
| 1 | 1 | EBCS.wCommand | BIT0 strobe command |
| 2 | 2 | EBCS.dwCodeFC | Command opcode |
| 4 | 1 | EBCS.byDataFC | Given if type byte |
| 5 | 1 | EBCS.wDataFC | Given if type word |
| 6 | 1 | EBCS.wDataFC_b |  |
| 7 | 2 | EBCS.dwDataFC | Given if type dword |
| 9 | 4 | EBCS.dDataFC | Dato se tipo double |
| 53 | 40 | EBCS.strDataFC[40] | Given if type string |

Output EsaBend Server:

| Address Modbus | N.Register 16 bit | Registers in shared memory | Description |
| --- | --- | --- | --- |
| 1024 | 2 | EBCS.dwErrorCode | Error code |
| 1026 | 100 | EBCS.strErUITextA[200] | Error description header |
| 1126 | 100 | EBCS.strErUITextB[200] | Error description |
| 1226 | 1 | EBCS.wAckMode | Acknowledgement operative modes: BIT0 Editor BIT1 Manual BIT2 SemiAutomatic BIT3 Automatic |
| 1227 | 1 | EBCS.wActualMode | Active operating mode: BIT0 Editor mode active BIT1 Manual mode active BIT2 Semiautomatic mode active BIT3 Automatic mode active |
| 1228 | 1 | EBCS.wErrData | Windows popup error code |
| 1229 | 2 | EBCS.dwErrValue |  |
| 1231 | 1 | EBCS.wAckCommandFS | BIT0 acknowledgement command |
| 1232 | 1 | progS.typeS | Type of bend |
| …. | …. | …. | …. |
| …. | …. | …. | … |
| 1965 | 82 | nprS | Program name |

To debug the protocol .

## Timed reading to avoid disconnection

In the event of a lack of communication, the server disconnects but to avoid this, the client simply needs to read an input register every 10 seconds.

## Disable commands and data from supervisor

Reception of commands or data from the supervisor is disabled (EsaBend does not execute the command and does not modify the data) if bit C44.15 is set to 1 or the "Disable commands and data from supervisor" parameter is set on the IoT page:

## Disable data for supervisor

Sending data to the supervisor is disabled (EsaBend does not refresh the data for the supervisor) if bit C44.16 is set to 1 or the "Disable data for supervisor" parameter is set on the IoT page:

## Data format used in ModBus protocol

### Formato WORD 16bit

The 16bit format is the minimum format used by the ModBus protocol; the byte order used by the modbus protocol is big-endian, from the most significant byte to the least significant byte.

### Formato DWORD 32bit

32bit data is stored in two consecutive registers starting from the even address.

|  | Address |
| --- | --- |
| HIGH REGISTER | 0 |
| LOW REGISTER | 1 |

The byte order used by the ModBus protocol is big-endian, from most significant to least significant byte.

### String format

String format fields are stored on consecutive registers, two bytes for each register; the maximum limit of registers read in block is 125 and the maximum limit of registers written in block is 123. The table below shows an example of how a 10 character ASCII string will be stored.

Es: String “0123456789”

|  | Address | Value |
| --- | --- | --- |
| REG. | 0 | 0x3031 |
| ……... | 1 | 0x3233 |
| …….. | … | 0x3435 |
|  |  | 0x3637 |
| REG. | 4 | 0x3839 |

The byte order used by the Modbus protocol is big-endian, from most significant to least significant byte.

## Log

The feature allows you to trace commands and data coming from an external supervisor

### Enable

In the Ppg.ini file under the [APP] section add the keys IOT_LOG=1

LOG_FILE=1

### Log file

In

C:\KVFILE\setupkv\ EsaBendLog.txt there is log file

The Log file is reset every time EsaBend is restarted; in case of malfunction, copy and send EsaBendLog.txt to Esautomotion BEFORE restarting EsaBend.

## Test con Client EsaBendClient.exe

To perform connection, command, data writing and data reading tests from clients, the EsaBendClient.exe application can be used, see EsaBendClient- Rev.0 - ENG.pdf document.

## EsaBend ModBusTCP connection configuration

### Description

The connection configuration between supervisor and Esabend ModBusTCP server is in the ServerModBus.XML file; the default written configuration is localost (127.0.0.1), i.e. the supervisor on the same PC as EsaBend. If you want to modify it, you need to change the ServerModBus.XML file and the window network configuration.

### Supervisor PC physical connection with EsaBend PC

#### Direct connection

Corporate Network

CNC

EsaBend PC

Supervisor PC

    - For EsaBend PC configuration use the following IP addres, IP address: 172.16.0.51, Subnet mask: 255.255.0.0

    - EsaBend PC in C:\Kvara\exe\ServerModBus.XML set the key <IPAddr>172.16.0.51</IPAddr>

    - For Supervision PC configuration use the following IP addres, IP address: 172.16.0.50, Subnet mask: 255.255.0.0

To configure: Start->Network Connections (Fig.1)

Fig.1

Right click on Len3 (or Len 2) Fig.2

Fig.2

General, Internet Protocol (TCP\IP), Properties (Fig.3)

Fig.3

- For EsaBend PC configuration use the following IP addres, IP address: 172.16.0.51, Subnet mask: 255.255.0.0 (Fig.4)

#### Fig.4

- For Supervision PC configuration use the following IP addres, IP address: 172.16.0.50, Subnet mask: 255.255.0.0 (Fig.5)

#### Fig.5

  - Direct connections with hub

# Corporate Network

Supervisor PC

CNC

EsaBend PC

HUB

For EsaBend PC configuration use the following IP addres, IP address: 10.10.51.99, Subnet mask: 255.255.0.0

- EsaBend PC in C:\Kvara\exe\ServerModBus.XML set the key <IPAddr>10.10.51.99<IPAddr>

- For Supervision PC configuration use the following IP addres, IP address: 10.10.51.51, Subnet mask: 255.255.0.0

  - Connection with corporate network

# Corporate Network

Supervisor PC

CNC

EsaBend PC

To set the correct IP in C:\Kvara\exe\ServerModBus.XML check:

    - For EsaBend PC configuration use the IP addres, where the first two numbers of the address are the same corporate network address. This static IP must be set also in the corporate server (ex. 192.168.40.30)

    - In EsaBend PC in C:\Kvara\exe\ServerModBus.XML set the key <IPAddr>192.168.40.30<IPAddr>

    - The Supervision PC configuration IP addres is dynamically assigned by the corporate server

    - The Supervisor Client must connect (in this example) 192.168.40.30

## Antivirus

Many antiviruses block the channel that uses ModBusTCP so it is necessary to either configure the antivirus so that it does not block communication or completely stop the antivirus.

Also disable the "Windows Defender Fire Wall" on the PC where EsaBend resides:

## 15. Directly automatically

When loading the program from the supervisor, EsaBend goes directly into automatic mode. If you don't want to log in automatically, you need to set the key below in PPG.ini [ModBusTCPClient]

NO_AUTO=0

Andrea Turchi

Rev.2 09/08/2024 Rev.1 08/05/2024 Rev.0 26/06/2024